Enclosure 2

INLAND WATER SMALL PASSENGER BOAT LICENSE REQUIREMENTS FOR PASSENGER BOATS NOT EXCEEDING 12 PERSONS WITH CREW

Boat:_______________________	License No___________   Date:___________

- Management:

  - Documentary proof of risk assessments/safety procedures

(Emergency response plan detailing procedures for calling Ambulance/Fire Brigade/Coast Guard etc								---

  - Insurance- Passenger liability min £3,000,000					---

  - Demonstrate recent history of safe working					---

- Hull Inspection:

a.	Steering including rudder, tiller, bearings, propeller, hydraulics/cables emergency provision (oars/spare tiller)						---

b.	Anodes										---

c.	Hull fittings internal & external							---

d.	Hull condition internal & external –stringers, engine mounts, plates		---

- Machinery:

a.	Engines boxed, fire port, sealed and insulated. 					---

b.	Exhausts lagged, spray shields, no glass fuel bowls				---

c.	Fuel cut off out side engine bay							---

d.	Fuel filler at or above tank ht with flame gauze					---

e.	Fuel tank sight glasses- v/v’s each end if used					---

- Electrical:

a.	Earthing straps on Tanks/ Engines to hull					---b.	Batteries x 2 change over switch fully chargeable			           ---

c.	Batteries well secured and in well ventilated space				---

d.	Safety equipment (VHF/Bilge pumps/ lighting) should run when engine off  ---

e.	Deck lighting if available- alternative waterproof torch				---

- Steering:

- Effective means of steering							---

- Clear view from steering position for navigation					---

- Emergency steering arrangements (Oar/ tiller on rudder head)	           ---

- Bilge Pumping/Draining:

a.	Powered or hand operated suitable for size of boat				---

b.	Every compartment drainable							---

c.	Auto start recommended checked regularly (not in oily bilge areas)		---

d.	Bilge alarms recommended – audible & visual preferred near controls	---

e.	If situated in oily areas- use absorbent pads under pollutant source		---

- Life saving Appliances:

a.	Minimum 2 suitable lifebuoys one with 18m min buoyant line attached 	---

b.	If night operations one with a light							---

c.	Enough life jackets for all passengers in an emergency			---

(life jackets- solid or inflatable type and approved (MCA/DfT/Wheelmarked))

Life jackets relying totally on oral inflation not appropriate unless inflated at all times.

d.	If vessel operates at night life jackets must have lights attached		---

e.	Instruction manual for onboard maintenance can be kept ashore to include

the following:  Check list; maintenance & repair instructions; log of records and inspections; schedule of periodic maintenance				---

- Fire Safety:

a.	Machinery boundaries adequate to retain F/F medium				---

b.	Portable fire extinguisher for engine fire min rating 5A/34B			---

c.	At least 1 multi purpose fire extinguisher min rating 13A/113B or more

smaller ones giving same fire rating to fight non engine fires			---

d.	One or more fire buckets with lanyards						---

- Communication Equipment:

- Waterproof VHF (advisable)							---

- Mobile phone in waterproof jacket (acceptable)					---

- Card giving clear summary of the distress/safety procedures displayed in view of comms equipment								---

- Navigation Lights:

a.	Nav lights- only required if operating between sunset/sunrise 	    Yes/No

b.	Waterproof Torch – mandatory							---

11. Other Safety Equipment:

a.	Boat Hook										---

b.	Suitable Tool Kit									---

c.	First Aid Kit										---

d.	Suitable Anchor and cable								---

12. License Details:

A notice stating the following information; the recommended height of the letters/figures is 2.5cm but, providing they are clear, they may be smaller:

- SHDC Licence Number and name of vessel

- Maximum number of persons to be carried

- Area of operation

- Name of owner/operator								---

Lockers containing the first aid kit, lifejackets and all emergency apparatus should be clearly marked											---

- Manning- Skipper:

Minimum qualifications:

- General- Competent boat handler & able to deal confidently with pass      	---

- Comms- VHF 1 day or able to use any available equipment

(phone loud hailer)									---

- Medically Fit   (ML5)									---

- BSSC- recommended									---

- LSA- Demonstrate position & usage of LSA onboard				---

- 1st Aid Course- Valid 1st aid certificate (valid for 5 yrs)				---

- Stability Heel Test:

7m Boat or Under:

- Minimum upright freeboard when fully loaded (12 passengers + 1 crew)                    240mm

- Maximum heeling angle with all 12 passengers on one side (must do both sides)    7 deg

- If over 7 deg but under 10 deg then as long as the minimum freeboard is above 240mm then this is classed as a pass

18m Boat or Over:

- As above but minimum upright freeboard when fully loaded  equal to 480mm and hence if heel is over 7 deg but 10 deg or less then the heeled freeboard must be greater than 480mm

7- 18m Boats:

Interpretation between 240mm for 7m boat & 480mm for 18m plus boat. Therefore:

Length 		Loaded Upright Freeboard

8m				260mm

9m				285mm

10m			305mm

11m			330mm

12m			350mm

13m			370mm

14m			395mm

15m			415mm

16m			440mm

17m			460mm

Hence a 10m boat, when fully loaded should have a freeboard of 305mm or more and when all the passengers are on one side could heel over to 10deg as long as the heeled freeboard was more than or equal to 305mm. If the heeled angle was less than or equal to 7deg then the freeboard does not matter.

Required		Actual

- Length of boat in metres					   	       -------

- Minimum loaded  upright freeboard in mm		---------			--------

- Loaded heeled angle Port side in degrees		---------			--------

- Loaded heeled angle Starboard side in degrees	---------			--------

- If over 7deg but 10 deg or less; freeboard Port in mm	---------			--------

- If over 7deg but 10 deg or less; freeboard Stb in mm	---------			--------

- If under required freeboard then reduction in passenger numbers to achieve 10degs and minimum heeled loaded freeboard.				       -------

Pass or Fail Stability/Heel test					---------------------

Recommendations: